EMBEDDED SYSTEMS

ASSIGNMENT 02

SUBMITTED BY:

Fahad Khan :   23-SE-64

SUBMITTED TO:

DR. HUMA QAYYUM

DEPARTMENT OF SOFTWARE ENGINEERING.

UNIVERSITY OF ENGINEERING AND TECHNOLOGY

TAXILA

# QUESTION 01 :  DUAL SWITCH SECURITY ALERT SYSTEM

SOURCE CODE :

.include "m32def.inc"

; ==========================================

; INITIALIZATION

; ==========================================

; 1. Initialize Stack Pointer (Best Practice) ldi r16, high(RAMEND) out SPH, r16 ldi r16, low(RAMEND) out SPL, r16

; 2. Configure Ports ldi r16, 0x00 out DDRA, r16     ; Configure Port A as Input

ldi r16, 0xFF out DDRB, r16     ; Configure Port B as Output out DDRC, r16     ; Configure Port C as Output

; ==========================================

; MAIN LOOP

; ========================================== start:

in r16, PINA      ; Read the current state of Port A     andi r16, 0x03    ; Mask to only look at PA1 and PA0 (0b00000011)

; Condition 1: Both Active (1,1) -> SAFE -> All OFF     cpi r16, 0x03

breq safe_state

; Condition 2: PA1 is 1, PA0 is 0 -> Turn ON LED1

cpi r16, 0x02          breq led1_on

; Condition 3: PA1 is 0, PA0 is 1 -> Turn ON LED2

cpi r16, 0x01          breq led2_on

; Condition 4: Both Inactive (0,0) -> Both LEDs ON + Buzzer ON

cpi r16, 0x00          breq critical_state

rjmp start        ; Fallback loop

; ==========================================

; STATE SUBROUTINES

; ========================================== safe_state:

cbi PORTB, 0      ; Clear PB0 (LED1 OFF)     cbi PORTB, 1      ; Clear PB1 (LED2 OFF)     cbi PORTC, 0      ; Clear PC0 (Buzzer OFF)     rjmp start

led1_on:     sbi PORTB, 0      ; Set PB0 (LED1 ON)     cbi PORTB, 1      ; Clear PB1 (LED2 OFF)     cbi PORTC, 0      ; Clear PC0 (Buzzer OFF)     rjmp start

led2_on:     cbi PORTB, 0      ; Clear PB0 (LED1 OFF)     sbi PORTB, 1      ; Set PB1 (LED2 ON)     cbi PORTC, 0      ; Clear PC0 (Buzzer OFF)     rjmp start

critical_state:

sbi PORTB, 0      ; Set PB0 (LED1 ON)     sbi PORTB, 1      ; Set PB1 (LED2 ON)     sbi PORTC, 0      ; Set PC0 (Buzzer ON)     rjmp start

SCHEMATIC PICTURE :

# QUESTION 02 :  SIMPLE SILENCE INDICATOR SYSTEM

SOURCE CODE :

.include "m32def.inc"

; ==========================================

; INITIALIZATION

; ==========================================

; Initialize Stack Pointer (Best Practice) ldi r16, high(RAMEND) out SPH, r16 ldi r16, low(RAMEND) out SPL, r16

; Configure Ports ldi r16, 0x00 out DDRA, r16     ; Configure Port A as Input (PA0 for the button)

ldi r16, 0xFF out DDRB, r16     ; Configure Port B as Output (PB0 for Green, PB1 for Red) out DDRC, r16     ; Configure Port C as Output (PC0 for Buzzer)

; ==========================================

; MAIN LOOP

; ========================================== start:

in r16, PINA      ; Read the current state of Port A     andi r16, 0x01    ; Mask to only look at PA0 (0b00000001)

; Check Condition: Button = 1 (Silence Mode)

cpi r16, 0x01          breq silence_mode

; If it's not 1, it must be 0 (Noise Condition)     rjmp noise_mode

; ==========================================

; STATE SUBROUTINES

; ========================================== silence_mode:     sbi PORTB, 0      ; Set PB0 (Green LED ON)     cbi PORTB, 1      ; Clear PB1 (Red LED OFF)     cbi PORTC, 0      ; Clear PC0 (Buzzer OFF)     rjmp start

noise_mode:     cbi PORTB, 0      ; Clear PB0 (Green LED OFF)     sbi PORTB, 1      ; Set PB1 (Red LED ON)     sbi PORTC, 0      ; Set PC0 (Buzzer ON)     rjmp start

SCHEMATIC PICTURE :

# QUESTION 03 : TWO LEVEL WARNING INDICATOR SYSTEM

SOURCE CODE :

.include "m32def.inc"

; ==========================================

; INITIALIZATION

; ========================================== ldi r16, high(RAMEND) out SPH, r16 ldi r16, low(RAMEND) out SPL, r16

ldi r16, 0x00 out DDRA, r16     ; Configure Port A as Input (PA0, PA1 for Logic Inputs)

ldi r16, 0xFF out DDRB, r16     ; Configure Port B as Output (PB0, PB1 for LEDs) out DDRC, r16     ; Configure Port C as Output (PC0 for Buzzer)

; ==========================================

; MAIN LOOP

; ========================================== start:

in r16, PINA      ; Read Port A     andi r16, 0x03    ; Mask to only look at PA1 and PA0

; (0,0) -> Normal -> All OFF     cpi r16, 0x00          breq state_normal

; (1,0) -> Level 1 Warning -> LED1 ON (Note: PA0 is 1, PA1 is 0)

cpi r16, 0x01          breq state_level1

; (0,1) -> Level 2 Warning -> LED2 ON (Note: PA0 is 0, PA1 is 1)

cpi r16, 0x02          breq state_level2

; (1,1) -> Critical -> Both LEDs ON + Buzzer ON

cpi r16, 0x03          breq state_critical

rjmp start

; ==========================================

; STATE SUBROUTINES

; ========================================== state_normal:     cbi PORTB, 0      ; LED1 OFF     cbi PORTB, 1      ; LED2 OFF     cbi PORTC, 0      ; Buzzer OFF     rjmp start

state_level1:     sbi PORTB, 0      ; LED1 ON     cbi PORTB, 1      ; LED2 OFF     cbi PORTC, 0      ; Buzzer OFF     rjmp start

state_level2:     cbi PORTB, 0      ; LED1 OFF     sbi PORTB, 1      ; LED2 ON     cbi PORTC, 0      ; Buzzer OFF     rjmp start

state_critical:

sbi PORTB, 0      ; LED1 ON     sbi PORTB, 1      ; LED2 ON     sbi PORTC, 0      ; Buzzer ON     rjmp start

SCHEMATIC PICTURE :

# QUESTION 04 : SMART ENTRY INDICATOR SYSTEM

SOURCE CODE :

.include "m32def.inc"

; ==========================================

; INITIALIZATION

; ========================================== ldi r16, high(RAMEND) out SPH, r16 ldi r16, low(RAMEND) out SPL, r16

ldi r16, 0x00 out DDRA, r16     ; Configure Port A as Input (PA0 for Button)

ldi r16, 0xFF out DDRB, r16     ; Configure Port B as Output (PB0, PB1 for LEDs) out DDRC, r16     ; Configure Port C as Output (PC0 for Buzzer)

clr r19           ; r19 will act as our "Seconds" counter

; ==========================================

; MAIN LOOP

; ========================================== start:     in r16, PINA           andi r16, 0x01         cpi r16, 0x01          breq button_pressed

; --- State: Button is NOT pressed --- button_released:

clr r19           ; Instantly reset the long-press counter to 0     cbi PORTB, 0      ; LED1 OFF     sbi PORTB, 1      ; LED2 ON (Entry Blocked)     cbi PORTC, 0      ; Buzzer OFF     rjmp start

; ==========================================

; STATE: BUTTON IS HELD DOWN

; ========================================== button_pressed:     sbi PORTB, 0      ; LED1 ON (Entry Allowed)     cbi PORTB, 1      ; LED2 OFF

; Check if we have held it long enough (The Threshold)     cpi r19, 20       ; <--- CHANGE THIS NUMBER TO ADJUST THE DELAY TIME

brsh trigger_buzzer

; If we haven't reached the threshold, add 1 to our counter     inc r19

; --- The Delay Block (Wastes time so it doesn't count instantly) ---     ldi r21, 255      ; Outer loop counter delay_outer:

ldi r20, 255      ; Inner loop counter

delay_inner:

; CRUCIAL: Constantly check if they let go of the button during the delay!

in r16, PINA     andi r16, 0x01     cpi r16, 0x01     brne button_released ; If released early, abort the delay and reset!

dec r20     brne delay_inner     dec r21     brne delay_outer

; -------------------------------------------------------------------

rjmp start        ; Go back to the top to check again

trigger_buzzer:     sbi PORTC, 0      ; Buzzer ON     rjmp start        ; Keep looping (if button is released, it jumps to 'button_released')

SCHEMATIC PICTURE :